# PRESS RELEASE

Würth Elektronik ICS presents 6 Way Symbol Cluster CAN

Compact, CAN-based indicator solution for mobile machines and commercial vehicles

Niedernhall (Germany), 13 January 2021 – Würth Elektronik ICS has complemented its product portfolio of human machines interfaces with a new, compact and easy to install symbol cluster with customisable front-end design. It can be used in commercial vehicles and mobile machines for indication of vehicle’s signals or additional functions.

Measuring 24 x 97 x 20 mm and weighing just about 20 g, the compact 6 Way Symbol Cluster CAN module fits easily into small vehicle or machine cabs. It can be operated in a voltage range of 8 to 32 VDC, and requires between 25 and 100 mA when in full use. The module has an operating temperature range of -30 °C to +80 °C.

The module offers 6 LEDs, a digital ‚low side‘ output and a digital input, all are freely addressable from the CAN bus. Thanks to the use of polyester membrane as a front layer, the symbols and colors can be customised, on demand.

Numerous configuration options

The CAN-based 6 Way Symbol Cluster can be configured using WE Flasher Tool, a configuration application from Würth Elektronik ICS which is used to upload the settings onto the device. Boot-up LED activation, CAN timeout behaviour and LED dimming/activation are amongst of the parameters that can be set. The 150 mA low-active output could be used to mute audio systems, activate buzzers or backlight a rocker switch. The 6 Way Symbol Cluster CAN can be easily integrated into an existing CAN network. This can be done using available signals, or used as a slave to an electronic control unit (ECU).

Thanks to fully programmable CAN communication with an 11- or 29-bit identifier, a freely configurable bit position for LED control and a selectable bus speed of 100/125/250/500/1000 kbps, the display module is well-suited for a wide variety of applications. A MPC3 connector with a mechanical lock on the back of the module assures a reliable connection.

Personalised service and advice

The 6 Way Symbol Cluster CAN is part of the WEcabin Solutions product line from Würth Elektronik ICS. WEcabin Solutions are high-quality HMI (human machine interface) solutions used to control and operate vehicle and comfort functions in mobile machines and commercial vehicles. Combined with our ICCS controllers and Central Electrical Units, they provide customers all the benefits of highly functional all-in-one solutions from a single source.

What truly sets the WEcabin Solutions product line apart is the service you receive with it, which includes standard personalised on-site support and advice. The team of field application engineers at Würth Elektronik ICS are there to assist customers when programming and configuring the products.

Available images

The following images can be downloaded from the Internet in printable quality: http://www.htcm.de/kk/wuerth_ics

| Image source: Würth Elektronik ICS The CAN-based 6 Way Symbol Cluster (Front) | Image source: Würth Elektronik ICS A MPC3 connector with a mechanical lock on the back of the CAN-based indicator solution assures a reliable connection. |
| --- | --- |

About Würth Elektronik ICS GmbH & Co. KG

Würth Elektronik ICS is a system supplier for electromechanical and electronic solutions for signal and power distribution, control, and display and operating solutions. ICS has been on the market as a member of the Würth Elektronik Group since 1984, and currently has about 400 employees, who generate an annual turnover of over €75 million. Its company headquarters are located in Niedernhall, Germany. There are subsidiaries of the company in France, the U.S., and India. Its main customers include well-known manufacturers of construction and agricultural machinery, as well as commercial vehicles. Sectors such as industrial technology or renewable energies also benefit from products and services provided by ICS.

Würth Elektronik: more than you expect!

Further info at www.we-online.com/ics

| More information: Würth Elektronik ICS GmbH & Co. KG Sandra Herter Gewerbepark Waldzimmern Würthstrasse 1 74676 Niedernhall Germany Phone: +49 7940 9810-1503 E-mail: sandra.herter@we-online.de www.we-online.com/ics | Press contact: HighTech communications GmbH Brigitte Basilio Brunhamstrasse 21 81249 Munich Germany Phone: +49 89 500778-20 Telefax: +49 89 500778-77 E-mail: b.basilio@htcm.de www.htcm.de |
| --- | --- |